Instructions on the XML phonebook of Yealink phones

Overview

XML Phonebook enable the phone to access to an XML-formatted phonebook which is located on an HTTP\TFTP server.

There are two phonebook.xml files in the ZIP file, if the firmware you use is x.43.x.x or higher, you can use phonebook.43.xml file; If the firmware you use is x.42.x.x, please use phonebook42.xml file, or you can update your phone to v43 firmware which on the following addresses:

T28: 	http://www.yealink.com/fae/V43/2.43.0.50.rar

T26: 	http://www.yealink.com/fae/V43/6.43.0.50.rar

T22: 	http://www.yealink.com/fae/V43/7.43.0.50.rar

T12: 	http://www.yealink.com/fae/V43/5.43.0.50.rar

The difference between v42 and v43 is:

A section between <DirectoryEntry> and </DirectoryEntry> defines just one contact in the version x.42.x.x, and it can define a number of contacts in the version x.43.x.x or higher.

Follow the steps below to use the function

- Prepare an XML file.

Follow the template file phonebook.xml. The file can be edited by UltraEdit or Windows Notepad. You can change the words which marked with red font, do not change the words with black font which are the fixed format for the xml phonebook.

Open the phonebook.xml and you will see it like this:

﻿ <xxxIPPhoneDirectory> <DirectoryEntry> <Name>Jam</Name> <Telephone>103</Telephone> <Telephone>104</Telephone> </DirectoryEntry> <SoftKeyItem> <Name>0</Name> <URL> http://192.168.0.18:8000/Test1.xml </URL> </SoftKeyItem> </xxxIPPhoneDirectory>

Pay attention to the following points:

- .<xxxIPPhoneDirectory> and </xxxIPPhoneDirectory>

You can change xxx to your company’s name, for example, </YealinkIPPhoneDirectory>.

- .The contact has its fixed format followed.

<DirectoryEntry> <Name>Jam</Name> <Telephone>103</Telephone> <Telephone>104</Telephone> </DirectoryEntry>

Note:  “<Telephone>number</Telephone>” support three formats.

E.g,

a.  <Telephone>444@intern.snom.com</Telephone>

b.  <Telephone>411</Telephone>

- <Telephone>sip:613@fwd.pulver.com</Telephone>

Please don’t change anything else except modifying the content between <name> and </name>, <Telephone> and </Telephone>.They define the name and the telephone number.

For more contacts, just add new sections similarly next to the other contact. When the file is done, give it a name, for example, phonebook.xml.

- .The section followed is for defining a sub-path when press the key(123456789*0#).

<SoftKeyItem> <Name>0</Name> <URL>http://192.168.0.18:8000/Test1.xml</URL> </SoftKeyItem>

- Put the XML file to your HTTP server.

Put the phonebook.xml on HTTP server, see the picture below, then the XML file is in this directory: http://192.168.0.18:8000/Phonebook.xml

- Open the web management of Yealink phones.

Click through “Contacts  Remote phone book”. Put the address into the box under “phone book url” and then give a name to it under “Phone book name”, for example ,we name it “Friends”. Confirm the settings, finally, it will be like:

- Enter to the Remote phonebook on the Phone.

- In the version x.42.x.x, turn to the phone LCD and look into address book and you will find a new item which is named “Friends”. You can access to that item to get the phonebook you’ve made.

- In the version x.43.x.x, when you press Directory on the IDLE LCD, it will display the  contacts which in the xml phonebook directly.

FAQ：

- when you enter to the xml phonebook, if it display “Load remote phonebook error”, please double check the below issue:

  - The format of the xml phonebook;

  - The URL is correct or not;

  - HTTP server is running or not.